Target: Journal of Agricultural and Food Chemistry (IF: 3.571)

Determination of geographical origin by multi-element profiling combined with machine learning techniques: a study on Chinese Geographical Indication (GI) rice

Fei Xu1, Fanzhou Kong1, Hong Peng1, Shuofei Dong2, Weiyu Gao1, Guangtao Zhang1*

1Mars Global Food Safety Center, Beijing 101407, China

2Agilent Technologies (China) Co., Ltd, Beijing 100102, China

*Corresponding author: Tel.: +86 13331152657. E-Mail: Guangtao.zhang@effem.com

ABSTRACT

This study aimed to develop a novel strategy to determine geographical origins of Chinese GI rice, based on multi-element profiling using inductively coupled plasma mass spectrometry (ICP-MS). One hundred and thirty-one samples from six types of Chinese GI rice were analyzed, with 80% of the data assigned as training set and the rest 20% as testing set. Two machine learning algorithms, support vector machines (SVM) and random forest (RF), along with feature selection (ReliefF algorithm) were implemented to build classification models. Independent validation by testing set showed that with the information from only four elements (27Al, 85Rb, 10B and 23Na), both SVM and RF could lead to a prediction accuracy of 100%. The results demonstrate that multi-element profiling combined with machine learning techniques is an effective strategy to determine the geological origins of rice.

KEYWORDS

rice, ICP-MS, Geographical Indication, machine learning, feature selection, chemometrics

INTRODUCTION

In the current world of globalization, geographical authentication of food materials has emerged as an important issue, which is crucial for both quality assurance and food safety1. The notion of geographical indication (GI) first originated from the 19th century in Europe, with the purpose of protecting industrial property rights2. Nowadays, products with GI certification generally possess given quality, reputations or other characteristics attributable to geographical origins3, thus making it possible for them to be differentiated from competitors. According to the European Union (EU) quality schemes, three different systems have been enforced to protect the origin integrity of agro products and food: protected designation of origin (PDO), protected geographical indication (PGI) and traditional specialties guaranteed (TSG)4. Currently in China, three government sectors supervise and protect GIs from different aspects at the administrative level5, including the State Administration for Industry and Commerce/ the Trademark Office (SAIC/TMO), the General Administration of Quality Supervision, Inspection and Quarantine (AQSIQ), and the Ministry of Agriculture (MoA).

Rice (Oryza sativa L.) is among the world’s top three largest food crops and is a staple food for nearly 50% of the world population6. China is leading in the rice paddy production in the world, with 220 million metric tons in 20187. With the improvement of people’s living standards, there is a growing domestic demand for rice with traceable origins8. However, due to the gap between the limited production and high market demand, GI rice has become more and more vulnerable to adulteration such as partial substitution and fraudulent labeling9,10. Therefore, determination of geographical origins of rice is of great importance for protecting the rights of farmers, retailers and consumers3.

Recent years, inductively coupled plasma mass spectrometry (ICP-MS) analysis in combination with multivariate analysis (MVA) and machine learning (ML) techniques, have been vigorously developed to address the issue of geographical authentication of rice11–13. ICP-MS is capable of simultaneously detecting both metal and non-metal elements, and is characterized by high throughput measurement with wide dynamic range and relatively simple sample preparation14. MVA has been generally used to process and integrate data with large volume, in which principal component analysis (PCA) and linear discriminate analysis (LDA) are the two dominant methods due to their simplicity in spotting hidden trends embedded in the dataset15. However, with the increase of data complexity, more advanced models based on pattern recognition are very much desired16. In the past decade, ML has demonstrated its capability to process complex problems, particularly in the domain of ecology17, biomedical18,19, astronomy20 and bioinformatics21. Yet so far, only a few studies have explored the application of ML in geographical authentication12,22.

The aim of this study was to develop a novel ML-based workflow for the determination of geographical origins of six types of Chinese GI rice. Support vector machines (SVM) and random forest (RF) were utilized to uncover the hidden information in the element profiling obtained by ICP-MS, and thereby to construct reliable prediction models. Furthermore, feature selection was also applied to identify biomarkers that contributed the most to the differentiation between GI rice.

MATERIALS AND METHODS

Rice samples

In this study, a total of one hundred and thirty-one Chinese GI rice samples were directly collected from credible rice processing factories in five provinces in China (Heilongjiang, Liaoning, Jiangsu, Hubei and Guangxi). For simplicity’s sake, we named these samples as WC, PJ-1, PJ-2, SY, JS and GG. An overview of the geographical information of the samples is shown in Fig. 1.

Fig.1. Geographical information of GI rice samples collected from different regions in China with the corresponding sample size.

Reagents and standards

Nitric acid (69%, part# 100441) was purchased from Merck Millipore (Darmstadt, Germany). Deionized water (18.3MΩ cm) was obtained from a Milli-Q system (Millipore, MA, USA). Multi-element calibration standard 2A (part# 8500-6940) and 4 (part# 8500-6942), environmental calibration standard (part# 5183-4688) and standard solutions of 45Sc (part# 5190-8578) and 103Rh (part# 8500-6945) were purchased from Agilent Technologies (Santa Clara, CA, USA). The certified reference material of rice flour (1568b) was purchased from the National Institute of Standards and Technology (Gaithersburg, MD, USA).

ICP-MS analysis

Firstly, 0.5 g of rice grains was weighed in a Teflon digestion vessel and mixed with 6mL of nitric acid. The vessel was placed in a fume hood overnight for pre-digestion and then transferred to the microwave oven (Anton Paar, Austria) for acid digestion. The digestion temperature of 180 °C was gradually reached in 15 min and held for 20 min. After digestion, the solution was cooled down to room temperature and diluted to 50mL with deionized water. To avoid cross contamination, all materials including the digestion vessels were soaked in a nitric acid solution (30% in water, v/v) for 24h and rinsed with deionized water for three times.

The concentrations of 30 elements (10B, 23Na, 24Mg, 27Al, 39K, 43Ca, 45Sc, 48Ti, 51V, 52Cr, 55Mn, 56Fe, 59Co, 60Ni, 65Cu, 66Zn, 70Ga, 73Ge, 75As, 78Se, 85Rb, 86Sr, 93Nb, 98Mo, 107Ag, 114Cd, 133Cs, 138Ba, 201Hg, 208Pb) were measured with Agilent 7900 ICP-MS (Agilent technologies, Santa Clara, CA, USA). The instrumental setting and operative conditions were adopted from a published method23 with some modifications. Briefly, under the helium tune mode, the plasma parameters were 1550 W radio frequency power, 8 mm sampling depth and 1.16 L·min-1 carrier gas flow (Argon). Cell gas (helium) flow of 5.0 mL·min-1 was also applied. The calibration solution was prepared by mixing and diluting the standards mentioned in the previous section (except for 103Rh). A diluted 103Rh standard solution (1 mg·L-1) was mixed with the sample stream, to correct matrix effects and compensate for the possible instrumental deviations. The accuracy of analysis was verified by analyzing the rice flour certified reference material once every ten samples. Each sample was analyzed in duplicate.

Statistical analysis

To preliminarily evaluate the concentrations of the 30 elements in each GI rice sample, the original dataset was analyzed by one-way analysis of variance (ANOVA) coupled with Tukey’s test (p < 0.05). The dataset was then scaled through logarithmic transformation and subjected to unsupervised PCA for initial visualization. Two machine learning algorithms, RF and SVM were implemented to construct classifiers respectively. RF was first introduced by Breiman24, and it is made of an ensemble of decision trees which are generated from the original dataset using bootstrap partition. SVM implements classifications by projecting input vectors into a high dimensional space thus finding a hyperplane that could separate different classes25. Feature selection is a data processing technique for data mining, which aims to identify pertinent features while discarding irrelevant ones that are not informative but contribute to the overall dimensionality of the problem space26. In this study, ReliefF27,28 was utilized to select features through investigating the relative importance based on a calculated proxy statistic28.Fig. 2 shows the workflow for the training of classifiers and the validation of classification models:

      - The scaled dataset was randomly split into a training set (sample number: 104) and a testing set (sample number: 27) in a stratified fashion (80:20).

      - The ReliefF algorithm was applied to the training set and all the 30 features were ranked based on their differentiation power. Subsequently, step-wise forward selection29 was conducted along with hyperparameter tuning (grid-search). After a 10-fold cross-validation, the best combinations of feature subsets and hyperparameters were used to construct optimal classifiers. The tested hyperparameters are shown in Table 1.

Table 1. Ranges of hyperparameters tested during model training.

| Hyperparameters | RF | SVM |
| --- | --- | --- |
| max_depth | [1, 11, 21, 31, 41, 51] | NA |
| max_features | ['auto', 'sqrt'] |  |
| n_estimators | [10, 120, 230, 340, 450, 560, 670, 780, 890, 1000] |  |
| Cost | NA | [0.1, 1, 10, 100, 1000] |

      - The optimal classifiers were then independently validated on the testing set, and the prediction accuracy was determined.

Fig.2. Diagram of the ML-based workflow for model training and validation.

All data analysis was carried out by R (R Core Team, 2019), RStudio (v3.5.1, Boston, MA, USA) and Python (v3.7, Python Core Team) with additional packages: dplyr (R)30, factoextra (R)31, FSelector (R)31, sklearn (Python)32, skrebate (Python)33, numpy (Python)34 and pandas (Python)35 .

RESULTS AND DISCUSSION

Elemental concentrations

As shown in Table S1, the measured concentrations of elements in the certified reference material agreed well with the certified values, indicating the high accuracy of the ICP-MS analysis. Table S2 shows the concentrations of the 30 elements measured in the 131 Chinese GI rice samples. Overall, except for 208Pb, significant differences were observed among all elements across all types of rice.

PCA analysis

As shown in Fig. 3a, the 1st and 2nd principal component (PC) were accountable for 60.7% of the total variance, and a clear separation was observed among PJ-1, GG and the rest types. While for JS, PJ-2, SY and WC, no satisfactory separation was achieved. The loading plot (Fig. 3b) showed that 27Al, 70Ga, 93Nb, 51V and 48Ti primarily contributed to the variations on PC1, while 23Na, 45Sc, 85Rb, 133Cs and 114Cd contributed to both PC1 and PC2. Notably, PJ-1 and PJ-2 could be clearly separated, despite that they were from the same geographical origin (Fig. 3a). A possible explanation could be that cultivar types could also contribute to the elemental composition in rice kernals11.

(a)

(b)

Fig.3. PCA analysis of the 30 elements measured in the 131 Chinese GI rice samples: (a) scoring plot of PC1 and PC2, with 95% confidence interval eclipse; (b) loading plot of all features (elements) on the first two PCs.

Determination of geographical origins

High-quality sampling is fundamental to achieve reliable results from multivariate modeling36. In this study, rather than sampling from the market, we obtained all GI rice samples from reliable sources, which ensured the authenticity of samples and minimized the risk of modeling with “contaminated” dataset. In addition, the numbers of samples of each rice type were quite similar, and  such a balanced dataset prevented the occurrence of misclassification due to modeling with imbalanced dataset12,37

To date, ML algorithms such as SVM and RF, have been reported to outperform traditional MVA38 as can build prediction models with superior reliability and robustness12,39. Fig. 4 shows the results obtained from the construction of optimal classifiers. Beyond the function of reducing dimensions, feature selection is also capable to identify features with high predictability, also known as biomarkers40. The relative importance of each of the 30 elements is shown in Fig. 4a, indicating that 27Al, 85Rb, 10B, 23Na and 86Sr were the leading elements that contributed the most to the differentiation of all types of GI rice. This is consistent with the observations in a previously published study, where feature selection was also applied and four elements (14Cd, 85Rb, 12Mg and 19K) among twenty-one elements were also found to be the most relevant for the differentiation between rice samples from two geographical origins12. The performance of both RF and SVM boosted significantly with more features added, as shown in Fig. 4b. With one feature (27Al), the mean cross-validation accuracy for RF and SVM were 48% and 63% respectively, both of which reached up to 100% as four features (27Al, 85Rb, 10B, and 23Na) were applied. Eventually, the optimal classifiers were constructed with four features and optimized hyperparameters (max_depth=37, max_features = ‘auto’, n_estimators = 200 for RF; ‘rbf’ kernel, with C value =1 and gamma value = 0.1 for SVM). Notably, in our study feature selection was applied solely on the training set rather than the entire dataset, which eliminated the risk of selection bias41.

(a)

(b)

Fig. 4. Feature ranking with ReliefF and model optimization with cross-validation (a) relative importance of each feature based on ReliefF; (b) cross-validation accuracy achieved by the classification models with different feature subsets.

While the goodness of fit of the model was assessed by cross-validation on the training set, the model performance should be ultimately validated on a dataset obtained from additional samples16.  According to the Principle of Proper Validation (PPV), independent validation using a separate dataset is the one and only valid paradigm for model assessment, since the testing set represents the future working situation of such particular model42. In our study, independent validation was conducted using the testing set and the result is shown in table 2. Both classifiers (RF and SVM) could make predictions of geographical origins with 100% accuracy for all six types of Chinese GI rice. The result demonstrated the effectiveness of the workflow established in this study, especially in constructing reliable prediction models while simultaneously identifying potential biomarkers responsible for the differentiation.

Table 2. Confusion matrix for the independent validation using the testing set.

| Classifier | Predicted | Reference |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | GG (n=3) | JS (n=4) | PJ-1 (n=7) | PJ-2 (n=4) | SY (n=4) | WC (n=4) |
| ReliefF + RF | GG | 3 | 0 | 0 | 0 | 0 | 0 |
|  | JS | 0 | 4 | 0 | 0 | 0 | 0 |
|  | PJ-1 | 0 | 0 | 7 | 0 | 0 | 0 |
|  | PJ-2 | 0 | 0 | 0 | 4 | 0 | 0 |
|  | SY | 0 | 0 | 0 | 0 | 4 | 0 |
|  | WC | 0 | 0 | 0 | 0 | 0 | 4 |
| ReliefF + SVM | GG | 3 | 0 | 0 | 0 | 0 | 0 |
|  | JS | 0 | 4 | 0 | 0 | 0 | 0 |
|  | PJ-1 | 0 | 0 | 7 | 0 | 0 | 0 |
|  | PJ-2 | 0 | 0 | 0 | 4 | 0 | 0 |
|  | SY | 0 | 0 | 0 | 0 | 4 | 0 |
|  | WC | 0 | 0 | 0 | 0 | 0 | 4 |

The differentiation power of the four features is clearly visualized in Fig. 5, where the relative median concentrations were plotted. It’s obvious that the element profile of each GI rice was significantly different. Particularly, the concentration of 27Al was the highest in PJ-1 whilst the lowest in PJ-2, even though PJ-1 and PJ-2 were sampled from the same geographical location. Besides, the other three elements also showed considerably different proportions in PJ-1 and PJ-2. These observations agreed well with previous findings that, cultivar types also played a significant role in the elemental composition of rice43,44. Due to the diversity and complexity of the sample set in this study, by now it’s still a challenge to elucidate exactly why the four elements showed such strong differentiation power. The samples were all from the three dominant rice-producing regions of China, including the northeast China plain (WC, PJ-1, and PJ-2), Yangtze River Basin (SY, JS) and southeast coastal region (GG). Such a geographically wide sampling scope introduced multiple layers of variables, including soil characteristics, agricultural practices and genotype variations, which could all contribute to the element profile of crops45,46.

Fig. 5. Radar plot of the relative median concentrations of the four features (27Al, 85Rb, 10B and 23Na ) in the six types of Chinese GI rice.

In a previously published study on Brazilian rice12, it has been reported that 114Cd only could differentiate rice samples from two geographical origins, and it was the difference in irrigation methods that resulted in the variance of 114Cd content in rice. In this study, 114Cd was detected in all six types of Chinese GI rice, with the highest concentration in GG which was sampled from Guangxi province (Table 1). Moreover, based on a previous national-scale study, the concentrations of 114Cd in paddy soils varied significantly in different regions of China, particularly higher in southeast coastal regions such as Guangxi province. Therefore, in this study, the feasibility of using only 114Cd to differentiate between GG and non-GG rice samples was also evaluated. The original dataset was regrouped as GG and non-GG, and the developed ML-based workflow was applied. The result of feature selection indicated that 114Cd was the element with the highest relative importance, and the prediction accuracy of 100% was achieved with only 114Cd. These results yet again demonstrated the effectiveness of the developed ML-based workflow in building reliable prediction models, especially using the least number of features in relatively simple cases, which could greatly improve the working efficiency and productivity.

In summary, this study successfully developed an ML-based workflow which could differentiate six types of Chinese GI rice with extremely high accuracy, in combination with multi-element profiling using ICP-MS. Particularly, amongst thirty elements that we analyzed, four elements were proved to be sufficient for the differentiation. With the success we have achieved using ICP-MS, we would further explore the possibility of applying our workflow with alternative instrumentations for the determination of the rice geographical origins. Particularly, with the advantage of rapidness and low risk of analyte loss, X-ray fluorescence (XRF) spectrometry has been evaluated and proved to be a good candidate for the purpose of determination of geographical origins of agro products47.

ABBREVIATIONS USED

ANOVA, one-way analysis of variance; AQSIQ, General Administration of Quality Supervision, Inspection and Quarantine; EU, European Union; GI, geographical indication; ICP-MS, inductively coupled plasma mass spectrometry; LDA, linear discriminate analysis; ML, machine learning; MoA, Ministry of Agriculture; MVA, multivariate analysis; PC, principal component; PCA, principal component analysis; PDO, protected designation of origin; PGI, protected geographical indication; RF, random forest; SAIC, State Administration for Industry and Commerce; SVM, support vector machines; TMO, the Trademark Office; TSG, traditional specialties guaranteed.

ACKNOWLEDGMENT

We want to thank Dr. Di Wu from Yangtze Delta Region Institute of Tsinghua University for his tremendous support on sampling. We thank Ms. Si Lin and Hongwei Qiao for their industrious work on experimentation and documentation.

FUNDING SOURCES

This project was fully funded by Mars, Incoprated and Agilent Foudation.

REFERENCE

1.	Drivelos, S. A. & Georgiou, C. A. Multi-element and multi-isotope-ratio analysis to determine the geographical origin of foods in the European Union. TrAC - Trends Anal. Chem. 40, 38–51 (2012).

2.	Summary of the Paris Convention for the Protection of Industrial Property (1883). Available at: https://www.wipo.int/treaties/en/ip/paris/summary_paris.html.

3.	Luykx, D. M. A. M. & Ruth, S. M. Van. Food Chemistry An overview of analytical methods for determining the geographical origin of food products. 107, 897–911 (2008).

4.	PDO and PGI Agricultral products:a 14.2 billion euro turnover for over 800 products. Available at: https://ec.europa.eu/agriculture/quality/schemes/newsletter-2010_en.pdf.

5.	Li, Y. Protection of Geographical Indications in China. (2017). Available at: https://www.niuyie.com/protection-of-geographical-indications-in-china/.

6.	Chauhan, B. S., Jabran, K. & Mahajan, G. Rice Production Worldwide.

7.	Paddy rice production worldwide 2017-2018, by country,. Statista (2019). Available at: https://www.statista.com/statistics/255937/leading-rice-producers-worldwide/.

8.	Jin, S., Zhang, Y. & Xu, Y. Amount of information and the willingness of consumers to pay for food traceability in China. Food Control (2017). doi:10.1016/j.foodcont.2017.02.012

9.	Rodriguez, L., Hall, B., Avenue, S. G., Hall, G. & Street, S. W. Social trust and risk knowledge , perception and behaviours resulting from a rice tampering scandal Sela Sar. 5, 80–96 (2014).

10.	Veeck, G. Food Safety Concerns and Rice Imports in China : 1998 - 2016. (2017).

11.	Cheajesadagul, P., Arnaudguilhem, C., Shiowatana, J., Siripinyanond, A. & Szpunar, J. Discrimination of geographical origin of rice based on multi-element fingerprinting by high resolution inductively coupled plasma mass spectrometry. Food Chem. 141, 3504–3509 (2013).

12.	Maione, C., Batista, B. L., Campiglia, A. D., Barbosa, F. & Barbosa, R. M. Classification of geographic origin of rice by data mining and inductively coupled plasma mass spectrometry. Comput. Electron. Agric. 121, 101–107 (2016).

13.	Wang, X. & Harrington, P. de B. Differentiating Rice Varieties by Inductively Coupled Plasma Mass Spectrometry Chemical Profiling with Singular Value Decomposition Background Correction. J. Anal. Test. 2, 138–148 (2018).

14.	Wadood, S. A., Boli, G., Xiaowen, Z., Hussain, I. & Yimin, W. Recent development in the application of analytical techniques for the traceability and authenticity of food of plant origin. Microchem. J. 152, 104295 (2020).

15.	Maione, C. & Barbosa, R. M. Recent applications of multivariate data analysis methods in the authentication of rice and the most analyzed parameters: A review. Crit. Rev. Food Sci. Nutr. 8398, 1–12 (2018).

16.	Jiménez-Carvelo, A. M., González-Casado, A., Bagur-González, M. G. & Cuadros-Rodríguez, L. Alternative data mining/machine learning methods for the analytical evaluation of food quality and authenticity – A review. Food Research International 122, (Elsevier Ltd, 2019).

17.	Cutler, D. R. et al. RANDOM FORESTS FOR CLASSIFICATION IN ECOLOGY. Ecology 88, 2783–2792 (2007).

18.	Hu, W. Identifying predictive markers of chemosensitivity of breast cancer with random forests. J. Biomed. Sci. Eng. 3, (2010).

19.	Ram, M., Najafi, A. & Shakeri, M. T. Classification and Biomarker Genes Selection for Cancer Gene Expression Data Using  Random Forest. Iran. J. Pathol. 12, 339–347 (2017).

20.	Gao, D., Zhang, Y. & Zhao, Y. Random forest algorithm for classification of multiwavelength data. Res. Astron. Astrophys. 9, 220 (2009).

21.	Boulesteix, A.-L., Janitza, S., Kruppa, J. & König, I. R. Overview of random forest methodology and practical guidance with emphasis on computational biology and bioinformatics. WIREs Data Min. Knowl. Discov. 2, 493–507 (2012).

22.	Richter, B., Rurik, M., Gurk, S., Kohlbacher, O. & Fischer, M. Food monitoring: Screening of the geographical origin of white asparagus using FT-NIR and machine learning. Food Control 104, 318–325 (2019).

23.	Hopfer, H., Nelson, J., Collins, T. S., Heymann, H. & Ebeler, S. E. The combined impact of vineyard origin and processing winery on the elemental profile of red wines. Food Chem. 172, 486–496 (2015).

24.	Breiman, L. Random Forests. Mach. Learn. 45, 5–32 (2001).

25.	Cortes, C. & Vapnik, V. Support-Vector Networks. Mach. Learn. 20, 273–297 (1995).

26.	Urbanowicz, R., Meeker, M., LaCava, W., Olson, R. & Moore, J. Relief-Based Feature Selection: Introduction and Review. J. Biomed. Inform. 85, (2017).

27.	Dietterich, T. Machine Learning Research: Four Current Directions. AI Mag. 18, (2000).

28.	Relief-based feature selection: Introduction and review. J. Biomed. Inform. 85, 189–203 (2018).

29.	Data Handling in Science and Technology. in Wavelets in Chemistry (ed. Walczak, B.) 22, ii (Elsevier, 2000).

30.	Wickham, H., François, R., Henry, L. & Müller, K. dplyr: A Grammar of Data Manipulation. (2019).

31.	Mundt, A. K. and F. factoextra: Extract and Visualize the Results of Multivariate Data Analyses. (2017).

32.	Pedregosa, F. et al. Scikit-learn: Machine Learning in {P}ython. J. Mach. Learn. Res. 12, 2825–2830 (2011).

33.	Urbanowicz, R. J., Olson, R. S., Schmitt, P., Meeker, M. & Moore, J. H. Benchmarking relief-based feature selection methods for bioinformatics data mining. J. Biomed. Inform. 85, 168–188 (2018).

34.	Walt, S. van der, Colbert, S. C. & Varoquaux, G. The NumPy Array: A Structure for Efficient Numerical Computation. Comput. Sci. Eng. 13, 22–30 (2011).

35.	McKinney, W. Data Structures for Statistical Computing in Python. in Proceedings of the 9th Python in Science Conference (eds. van der Walt, S. & Millman, J.) 51–56 (2010).

36.	Brereton, R. G. et al. Chemometrics in analytical chemistry—part I: history, experimental design and data analysis tools. Anal. Bioanal. Chem. 409, 5891–5899 (2017).

37.	Wei, Q. & Dunbrack Jr, R. L. The Role of Balanced Training and Testing Data Sets for Binary Classifiers in Bioinformatics. PLoS One 8, 1–12 (2013).

38.	Gromski, P. S. et al. A tutorial review: Metabolomics and partial least squares-discriminant analysis - a marriage of convenience or a shotgun wedding. Anal. Chim. Acta 879, 10–23 (2015).

39.	Devos, O., Ruckebusch, C., Durand, A., Duponchel, L. & Huvenne, J. Chemometrics and Intelligent Laboratory Systems Support vector machines ( SVM ) in near infrared ( NIR ) spectroscopy : Focus on parameters optimization and model interpretation ☆. Chemom. Intell. Lab. Syst. 96, 27–33 (2009).

40.	Grissa, D. et al. Feature Selection Methods for Early Predictive Biomarker Discovery Using Untargeted  Metabolomic Data. Front. Mol. Biosci. 3, 30 (2016).

41.	Krawczuk, J. & Łukaszuk, T. The feature selection bias problem in relation to high-dimensional gene data. Artif. Intell. Med. 66, 63–71 (2016).

42.	Esbensen, K. H. & Geladi, P. Principles of Proper Validation : use and abuse of re-sampling for validation. 168–187 (2010). doi:10.1002/cem.1310

43.	Li, Z., Li, L., Pan, G. & Chen, J. Bioavailability of Cd in a soil-rice system in China: Soil type versus genotype effects. Plant Soil 271, 165–173 (2005).

44.	Wang-da, C., Guo-ping, Z., Hai-gen, Y. A. O., Wei, W. U. & Min, X. U. Genotypic and environmental variation in cadmium , chromium , arsenic , nickel , and lead concentrations in rice grains *. 7, 565–571 (2006).

45.	Chung, I. M. et al. Geographic authentication of Asian rice (Oryza sativa L.) using multi-elemental and stable isotopic data combined with multivariate analysis. Food Chem. 240, 840–849 (2018).

46.	Zhang, Y., Song, Q., Yan, J. & Tang, J. Mineral element concentrations in grains of Chinese wheat cultivars. 303–313 (2010). doi:10.1007/s10681-009-0082-6

47.	Worku, M. et al. Differentiating the geographical origin of Ethiopian coffee using XRF- and ICP-based multi-element and stable isotope profiling. Food Chem. 290, 295–307 (2019).