Target: Journal of Agricultural and Food Chemistry (IF: 3.571)

Determination of geographical origin by multi-element profiling combined with machine learning techniques: a study on Chinese Geographical Indication (GI) rice

Fei Xu1, Fanzhou Kong1, Hong Peng1, Shuofei Dong2, Weiyu Gao1, Guangtao Zhang1*

1Mars Global Food Safety Center, Beijing 101407, China

2Agilent Technologies (China) Co., Ltd, Beijing 100102, China

*Corresponding author: Tel.: +86 13331152657. E-Mail: Guangtao.zhang@effem.com

ABSTRACT

This study aimed to develop a novel strategy to determine geographical origins of Chinese Geographical Indication (GI) rice, based on multi-element profiling using inductively coupled plasma mass spectrometry (ICP-MS). One hundred and thirty-one samples from six types of Chinese GI rice were analyzed; 80% of the data was assigned as a training set, and the remaining 20% were used as a test set. Two machine learning algorithms, support vector machines (SVM) and random forest (RF), together with feature selection (ReliefF algorithm) were implemented to build classification models. Independent validation using the testing set showed that with the information from only four elements (Al, Rb, B and Na), both SVM and RF could lead to a prediction accuracy of 100%. The results demonstrate that multi-element profiling combined with machine learning techniques is an effective strategy to determine the geographical origins of rice.

KEYWORDS

rice, ICP-MS, Geographical Indication, machine learning, feature selection, chemometrics

INTRODUCTION

Geographical authentication of food materials has emerged as an important issue in the current world of globalization, and is crucial for both quality assurance and food safety1. The notion of geographical indication (GI) first originated during the 19th century in Europe, with the purpose of protecting industrial property rights2. Nowadays, products with GI certification generally possess given quality, reputation or other characteristics attributable to their geographical origins3, thus making it possible for them to be differentiated from competitors. Three different systems are enforced under European Union (EU) quality schemes to protect the origin integrity of agro products and food: protected designation of origin, protected geographical indication and traditional specialties guaranteed4. Three government sectors supervise and protect different aspects of GIs at the administrative level in China5, including the State Administration for Industry and Commerce / the Trademark Office, the General Administration of Quality Supervision, Inspection and Quarantine, and the Ministry of Agriculture.

Rice (Oryza sativa L.) is among the world’s three largest food crops and is a staple food for nearly 50% of the world population6. China is the leader in rice paddy production in the world, producing 220 million metric tons in 20187. With the improvement of living standards, there is a growing domestic demand for rice with traceable origins8. However, due to the gap between limited production and high market demand, GI rice has become more and more vulnerable to adulteration, such as partial substitution and fraudulent labeling9,10. Determination of the geographical origins of rice is, therefore, of great importance for protecting the rights of farmers, retailers and consumers3.

Inductively coupled plasma mass spectrometry (ICP-MS) analysis in combination with multivariate analysis (MVA) has been vigorously developed in recent years to address the geographical authentication of rice11. ICP-MS is capable of simultaneously detecting both metal and non-metal elements, and is characterized by high throughput measurement with wide dynamic range and relatively simple sample preparation12. MVA has generally been used to process and integrate large data sets, in which principal component analysis (PCA) and linear discriminate analysis are the two dominant methods due to their simplicity in spotting hidden trends embedded in the dataset13. However, more advanced models based on pattern recognition are required, due to increased data complexity14. In the past decade, machine learning (ML) has demonstrated its capability to process complex problems, particularly in the domains of ecology15, biomedicine16,17, astronomy18 and bioinformatics19. Limited studies have explored the application of ML to the geographical authentication of rice13,20.

The aim of this study was to develop a novel ML-based workflow for the determination of geographical origins of six types of Chinese GI rice. Support vector machines (SVM) and random forest (RF) were used to uncover hidden information in the element profiling obtained from ICP-MS, thereby constructing reliable prediction models. Furthermore, feature selection was also applied to identify biomarkers that contributed most to the differentiation between types of GI rice.

MATERIALS AND METHODS

Rice samples

One hundred and thirty-one Chinese GI rice samples were directly collected from credible rice processing factories in five provinces in China (Heilongjiang, Liaoning [two sample sets], Jiangsu, Hubei and Guangxi). These are labelled as WC, PJ-1, PJ-2, SY, JS and GG respectively, in the remainder of this manuscript. Sample numbers obtained from each region are shown below (Figure 1).

Fig.1. Geographical origin and corresponding sample size of the Chinese GI rice samples.

Reagents and standards

Nitric acid (69%, part# 100441) was purchased from Merck Millipore (Darmstadt, Germany). Deionized water (DIW, 18.2MΩ·cm) was obtained from a Milli-Q system (Millipore, MA, USA). Multi-element calibration standard 2A (part# 8500-6940) and 4 (part# 8500-6942), environmental calibration standard (part# 5183-4688) and standard solutions of scandium (Sc, part# 5190-8578) and rhodium(Rh, part# 8500-6945) were purchased from Agilent Technologies (Santa Clara, CA, USA). The Standard Reference Material (SRM) of rice flour (1568b) was purchased from the National Institute of Standards and Technology (NIST, Gaithersburg, MD, USA).

ICP-MS analysis

0.5 g of rice grains was weighed in a polytetrafluoroethylene (PTFE) digestion vessel and mixed with 6mL of nitric acid. The vessel was placed in a fume hood overnight for pre-digestion and then transferred into the microwave oven (Anton Paar, Austria) for acid digestion. The digestion temperature of 180 °C was gradually reached in 15 min and held for 20 min. After digestion, the solution was cooled to room temperature and diluted to 50mL with DIW. All materials including the digestion vessels were soaked in a 30% (v/v) nitric acid solution for 24h and rinsed with DIW three times to avoid cross contamination.

The concentrations of 30 elements (B, Na, Mg, Al, K, Ca, Sc, Ti, V, Cr, Mn, Fe,  Co, Ni,  Cu, Zn, Ga, Ge, As, Se, Rb, Sr, Nb, Mo, Ag, Cd, Cs, Ba, Hg and Pb) were measured using an Agilent 7900 ICP-MS (Agilent technologies, Santa Clara, CA, USA). The instrumental setting and operating conditions were adopted from a previously published method21 with some modifications. Briefly, under the helium tune mode, the plasma parameters were 1550 W radio frequency power, 8 mm sampling depth and 1.16 L·min-1 carrier gas flow (Argon). Cell gas (helium) flow of 5.0 mL·min-1 was also applied. The calibration solution was prepared by mixing and diluting the standards mentioned in the previous section (except for Rh). A diluted Rh standard solution (1 mg·L-1) was mixed with the sample stream using a tee joint, to correct matrix effects and to compensate for possible instrument deviations. The accuracy of analysis was verified by analyzing the rice flour SRM 1568b once every ten samples. Each rice sample was analyzed in duplicate.

Statistical analysis

One-way analysis of variance (ANOVA) coupled with Tukey’s test (p < 0.05) was used for preliminary analysis of the concentration of the 30 elements in each GI rice. The dataset was then scaled through logarithmic transformation and subjected to unsupervised PCA for initial visualization. Two machine learning algorithms, RF and SVM, were implemented to construct classifiers. RF is an ensemble of decision trees which are generated from the original dataset using bootstrap partition22. SVM implements classifications by projecting input vectors into a high dimensional space, thus finding a hyperplane that could separate different classes23. Feature selection is a data processing technique for data mining which aims to identify pertinent features, while discarding irrelevant ones that are not informative, but contribute to the overall dimensionality of the problem space24. ReliefF24,25 was utilized to select features through investigating their relative importance based on a calculated proxy statistic. The workflow for the training of classifiers and the validation of classification models is shown below (Figure 2):

      - The scaled dataset was randomly split into a training set (n= 104) and a test set (n= 27) in a stratified fashion (80:20).

      - The ReliefF algorithm was applied to the training set and all the 30 features were ranked based on their differentiation power. Subsequently, step-wise forward selection26 was conducted along with hyperparameter tuning (grid-search). After a 10-fold cross-validation, the best combinations of feature subsets and hyperparameters were used to construct optimal classifiers. The tested hyperparameters are shown below (Table 1).

Table 1. Ranges of hyperparameters tested during model training.

| Hyperparameters | RF | SVM |
| --- | --- | --- |
| max_depth | [1, 11, 21, 31, 41, 51] | NA |
| max_features | ['auto', 'sqrt'] | NA |
| n_estimators | [10, 120, 230, 340, 450, 560, 670, 780, 890, 1000] | NA |
| Cost | NA | [0.1, 1, 10, 100, 1000] |

      - The optimal classifiers were then independently validated on the test set, and their prediction accuracies were determined.

Fig.2. Diagram of the ML-based workflow for model training and validation.

All data analysis was carried out using R (R Core Team, 2019), RStudio (v3.5.1, Boston, MA, USA) and Python (v3.7, Python Core Team) with additional packages: dplyr (R)27, factoextra (R)28, FSelector (R)28, sklearn (Python)29, skrebate (Python)30, numpy (Python)31 and pandas (Python)32 .

RESULTS AND DISCUSSION

Concentrations of elements

The measured concentrations of elements in the SRM 1568b agreed well with the certified values (recovery ranged from 80.8% to 102.3%), indicating the high accuracy of the ICP-MS analysis (Table S1). Results from analysis of the 30 elements in the 131 Chinese GI rice samples are shown in Table S2. Significant differences were observed among all elements across all types of rice, with the exception of Pb. However, such broadly observed difference in elemental concentrations were not able to predict which element(s) may contribute the most to the differentiation among six types of GI rice.

PCA analysis

The 1st and 2nd principal components (PCs) together accounted for 60.7% of the total variance, and a clear separation was observed among PJ-1, GG and the other types of GI rice (Fig. 3a). No satisfactory separation was achieved for JS, PJ-2, SY and WC. The loading plot (Fig. 3b) showed that Al, Ga, Nb, V and Ti primarily contributed to the variations in PC1, while Na, Sc, Rb, Cs and Cd contributed to both PC1 and PC2. Notably, PJ-1 and PJ-2 could be clearly separated, despite their common geographical origin (Fig. 3a).

(a)

(b)

Fig.3. PCA analysis of the 30 elements measured in the 131 Chinese GI rice samples: (a) scoring plot of PC1 and PC2, with 95% confidence interval eclipse; (b) loading plot of all features projected on the first two PCs.

Determination of geographical origins

High-quality sampling is fundamental to achieving reliable results from multivariate modeling33. In this study we collected all the rice samples from reliable sources rather than sampling from a market, which ensured the authenticity of samples and minimized the risk of modeling with a “contaminated” dataset. In addition, relatively equal quantities of each rice sample were collected, preventing the risk of misclassification due to modeling with an imbalanced dataset20,34

ML algorithms such as SVM and RF have been reported to outperform traditional MVA35, and can build predictive models with superior reliability and robustness20,36. The results from the construction of optimal classifiers are shown below (Fig. 4). Feature selection has the function of reducing dimensions, and is also capable of identifing features with high predictability, which are also known as biomarkers37. The relative importance of each of the 30 elements is shown below (Fig. 4a). The results indicated that Al, Rb, B, Na and Sr were the main elements that contributed to the differentiation of all types of GI rice. This is consistent with the observations in a previous study, where feature selection was also applied and four elements (Cd, Rb, Mg and K) out of 21 evaluated were found to be the most relevant for the differentiation between rice samples from two geographical origins20. The performance of both RF and SVM improved significantly as more features added were added (Fig. 4b). The mean cross-validation accuracies for RF and SVM were 48% and 63% respectively with one feature (Al), both reached 100% when four features (Al, Rb, B, and Na) were applied. The optimal classifiers were determined as four features with corresponding optimum hyperparameters (max_depth=37, max_features = ‘auto’, n_estimators = 200 for RF; ‘rbf’ kernel, with C value =1 and gamma value = 0.1 for SVM). Feature selection was applied solely to the training set and not to the entire dataset, which eliminated the risk of selection bias38.

(a)

(b)

Fig. 4. Feature ranking with ReliefF and model optimization with cross-validation: (a) relative importance of each feature based on ReliefF; (b) cross-validation accuracy of classification models built with different numbers of features.

Cross validation using a training set can assess the goodness of fit of a particular model. However, independent validation using a separate data set is critical to ultimately evaluate predictive performance, as it incorporates the future working situation39. Independent validation was conducted in this study using the test set (Table 2). Both classifiers (RF and SVM) could predict the geographical origins of all types of rice with 100% accuracy. The results demonstrated the effectiveness of the ML-based workflow established in this study, especially in constructing reliable predictive models, while simultaneously identifying potential biomarkers accounting for the differentiation.

Table 2. Confusion matrix for the independent validation using the test set.

| Classifier | Predicted | Reference |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | GG (n=3) | JS (n=4) | PJ-1 (n=7) | PJ-2 (n=4) | SY (n=4) | WC (n=4) |
| ReliefF + RF | GG | 3 | 0 | 0 | 0 | 0 | 0 |
|  | JS | 0 | 4 | 0 | 0 | 0 | 0 |
|  | PJ-1 | 0 | 0 | 7 | 0 | 0 | 0 |
|  | PJ-2 | 0 | 0 | 0 | 4 | 0 | 0 |
|  | SY | 0 | 0 | 0 | 0 | 4 | 0 |
|  | WC | 0 | 0 | 0 | 0 | 0 | 4 |
| ReliefF + SVM | GG | 3 | 0 | 0 | 0 | 0 | 0 |
|  | JS | 0 | 4 | 0 | 0 | 0 | 0 |
|  | PJ-1 | 0 | 0 | 7 | 0 | 0 | 0 |
|  | PJ-2 | 0 | 0 | 0 | 4 | 0 | 0 |
|  | SY | 0 | 0 | 0 | 0 | 4 | 0 |
|  | WC | 0 | 0 | 0 | 0 | 0 | 4 |

The differentiation power of the four features is visualized in the plot of relative median concentrations (Fig. 5). The elemental profile of each GI rice was significantly different. It is noteworthy that the concentration of Al was highest in PJ-1 and lowest in PJ-2, even though PJ-1 and PJ-2 were sampled from the same geographical location. In addition, the other three elements were present in considerably different proportions in PJ-1 and PJ-2. These observations agreed with previous findings that cultivar types also play a significant role in the composition of elements in rice kernals40,41. It remains a challenge to elucidate why the four elements showed such strong differentiation power, as the sample set used in this study was diverse and complex. The samples were from the three dominant rice-producing regions of China, including the northeast China plain (WC, PJ-1, and PJ-2), Yangtze River Basin (SY, JS) and southeast coastal region (GG). Such geographically wide sampling introduced multiple variables, including soil characteristics, agricultural practices and genotype variations, all of which could contribute to the elemental profile of crops42,43.

Fig. 5. Radar plot of the relative median concentrations of the four features (Al, Rb, B and Na) in the six types of Chinese GI rice.

A study on Brazilian rice20 reported that Cd only could differentiate rice samples from two geographical origins, and it was the difference in irrigation methods that resulted in the variance of Cd content. Cadmium was detected in all six types of Chinese GI rice, with the highest concentration in GG which was sampled from Guangxi province (Table S2). A previous national-scale study reported that the concentrations of Cd in paddy soils varied significantly in different regions of China, and were higher in southeast coastal regions such as Guangxi province. The feasibility of using only Cd to differentiate between GG and non-GG rice samples was also evaluated in this study. The original dataset was regrouped as GG and non-GG, and the developed ML-based workflow applied. The result of feature selection indicated that Cd was the element with the highest relative importance, and the prediction accuracy of 100% was achieved with Cd alone. These results again demonstrated the effectiveness of the ML-based workflow in building reliable predictive models. The workflow is effective in using the least number of features in relatively simple cases, potentially greatly improving working efficiency and productivity.

In summary, this study successfully developed a ML-based workflow which could differentiate six types of Chinese GI rice with extremely high accuracy when combined with multi-element profiling using ICP-MS. The study showed that a feature set of four elements was sufficient for differentiation. The workflow developed in this study could potentially be applied to other methods of elemental analysis to authenticate geographical origins of rice.

ABBREVIATIONS USED

ANOVA, one-way analysis of variance; GI, geographical indication; ICP-MS, inductively coupled plasma mass spectrometry; ML, machine learning; PC, principal component; PCA, principal component analysis; SVM, support vector machines;.

ACKNOWLEDGMENT

We want to thank Dr. Di Wu from Yangtze Delta Region Institute of Tsinghua University for his tremendous support on sampling. We thank Ms. Si Lin and Ms. Hongwei Qiao for their industrious work on experimentation and documentation.

FUNDING SOURCES

This project was fully funded by Mars, Incorporated and Agilent Foundation.

REFERENCE

(1) 	Drivelos, S. A.; Georgiou, C. A. Multi-Element and Multi-Isotope-Ratio Analysis to Determine the Geographical Origin of Foods in the European Union. TrAC - Trends Anal. Chem. 2012, 40, 38–51. https://doi.org/10.1016/j.trac.2012.08.003.

(2) 	Summary of the Paris Convention for the Protection of Industrial Property (1883) https://www.wipo.int/treaties/en/ip/paris/summary_paris.html.

(3) 	Luykx, D. M. A. M.; Ruth, S. M. Van. Food Chemistry An Overview of Analytical Methods for Determining the Geographical Origin of Food Products. 2008, 107, 897–911. https://doi.org/10.1016/j.foodchem.2007.09.038.

(4) 	PDO and PGI Agricultral products:a 14.2 billion euro turnover for over 800 products https://ec.europa.eu/agriculture/quality/schemes/newsletter-2010_en.pdf.

(5) 	Li, Y. Protection of Geographical Indications in China https://www.niuyie.com/protection-of-geographical-indications-in-china/.

(6) 	Chauhan, B.; Jabran, K.; Mahajan, G. Rice Production Worldwide; 2017. https://doi.org/10.1007/978-3-319-47516-5.

(7) 	Paddy rice production worldwide 2017-2018, by country, https://www.statista.com/statistics/255937/leading-rice-producers-worldwide/.

(8) 	Jin, S.; Zhang, Y.; Xu, Y. Amount of Information and the Willingness of Consumers to Pay for Food Traceability in China. Food Control 2017. https://doi.org/10.1016/j.foodcont.2017.02.012.

(9) 	Rodriguez, L.; Hall, B.; Avenue, S. G.; Hall, G.; Street, S. W. Social Trust and Risk Knowledge , Perception and Behaviours Resulting from a Rice Tampering Scandal Sela Sar. 2014, 5 (1), 80–96.

(10) 	Veeck, G. Food Safety Concerns and Rice Imports in China : 1998 - 2016. 2017, No. June.

(11) 	Cheajesadagul, P.; Arnaudguilhem, C.; Shiowatana, J.; Siripinyanond, A.; Szpunar, J. Discrimination of Geographical Origin of Rice Based on Multi-Element Fingerprinting by High Resolution Inductively Coupled Plasma Mass Spectrometry. Food Chem. 2013, 141 (4), 3504–3509. https://doi.org/10.1016/j.foodchem.2013.06.060.

(12) 	Wadood, S. A.; Boli, G.; Xiaowen, Z.; Hussain, I.; Yimin, W. Recent Development in the Application of Analytical Techniques for the Traceability and Authenticity of Food of Plant Origin. Microchem. J. 2020, 152, 104295. https://doi.org/10.1016/j.microc.2019.104295.

(13) 	Maione, C.; Barbosa, R. M. Recent Applications of Multivariate Data Analysis Methods in the Authentication of Rice and the Most Analyzed Parameters: A Review. Crit. Rev. Food Sci. Nutr. 2018, 8398, 1–12. https://doi.org/10.1080/10408398.2018.1431763.

(14) 	Jiménez-Carvelo, A. M.; González-Casado, A.; Bagur-González, M. G.; Cuadros-Rodríguez, L. Alternative Data Mining/Machine Learning Methods for the Analytical Evaluation of Food Quality and Authenticity – A Review. Food Res. Int. 2019, 122, 25–39. https://doi.org/https://doi.org/10.1016/j.foodres.2019.03.063.

(15) 	Cutler, D. R.; Edwards Jr., T. C.; Beard, K. H.; Cutler, A.; Hess, K. T.; Gibson, J.; Lawler, J. J. RANDOM FORESTS FOR CLASSIFICATION IN ECOLOGY. Ecology 2007, 88 (11), 2783–2792. https://doi.org/10.1890/07-0539.1.

(16) 	Hu, W. Identifying Predictive Markers of Chemosensitivity of Breast Cancer with Random Forests. J. Biomed. Sci. Eng. 2010, 3. https://doi.org/10.4236/jbise.2010.31009.

(17) 	Ram, M.; Najafi, A.; Shakeri, M. T. Classification and Biomarker Genes Selection for Cancer Gene Expression Data Using  Random Forest. Iran. J. Pathol. 2017, 12 (4), 339–347.

(18) 	Gao, D.; Zhang, Y.; Zhao, Y. Random Forest Algorithm for Classification of Multiwavelength Data. Res. Astron. Astrophys. 2009, 9, 220. https://doi.org/10.1088/1674-4527/9/2/011.

(19) 	Boulesteix, A.-L.; Janitza, S.; Kruppa, J.; König, I. R. Overview of Random Forest Methodology and Practical Guidance with Emphasis on Computational Biology and Bioinformatics. WIREs Data Min. Knowl. Discov. 2012, 2 (6), 493–507. https://doi.org/10.1002/widm.1072.

(20) 	Maione, C.; Batista, B. L.; Campiglia, A. D.; Barbosa, F.; Barbosa, R. M. Classification of Geographic Origin of Rice by Data Mining and Inductively Coupled Plasma Mass Spectrometry. Comput. Electron. Agric. 2016, 121, 101–107. https://doi.org/10.1016/j.compag.2015.11.009.

(21) 	Hopfer, H.; Nelson, J.; Collins, T. S.; Heymann, H.; Ebeler, S. E. The Combined Impact of Vineyard Origin and Processing Winery on the Elemental Profile of Red Wines. Food Chem. 2015, 172, 486–496. https://doi.org/10.1016/j.foodchem.2014.09.113.

(22) 	Breiman, L. Random Forests. Mach. Learn. 2001, 45 (1), 5–32. https://doi.org/10.1023/a:1010933404324.

(23) 	Cortes, C.; Vapnik, V. Support-Vector Networks. Mach. Learn. 1995, 20 (3), 273–297. https://doi.org/10.1023/A:1022627411411.

(24) 	Urbanowicz, R.; Meeker, M.; LaCava, W.; Olson, R.; Moore, J. Relief-Based Feature Selection: Introduction and Review. J. Biomed. Inform. 2017, 85. https://doi.org/10.1016/j.jbi.2018.07.014.

(25) 	Dietterich, T. Machine Learning Research: Four Current Directions. AI Mag. 2000, 18.

(26) 	Data Handling in Science and Technology. In Wavelets in Chemistry; Walczak, B., Ed.; Data Handling in Science and Technology; Elsevier, 2000; Vol. 22, p ii. https://doi.org/https://doi.org/10.1016/S0922-3487(13)70536-7.

(27) 	Wickham, H.; François, R.; Henry, L.; Müller, K. Dplyr: A Grammar of Data Manipulation. 2019.

(28) 	Mundt, A. K. and F. Factoextra: Extract and Visualize the Results of Multivariate Data Analyses. 2017.

(29) 	Pedregosa, F.; Varoquaux, G.; Gramfort, A.; Michel, V.; Thirion, B.; Grisel, O.; Blondel, M.; Prettenhofer, P.; Weiss, R.; Dubourg, V.; Vanderplas, J.; Passos, A.; Cournapeau, D.; Brucher, M.; Perrot, M.; Duchesnay, E. Scikit-Learn: Machine Learning in {P}ython. J. Mach. Learn. Res. 2011, 12, 2825–2830.

(30) 	Urbanowicz, R. J.; Olson, R. S.; Schmitt, P.; Meeker, M.; Moore, J. H. Benchmarking Relief-Based Feature Selection Methods for Bioinformatics Data Mining. J. Biomed. Inform. 2018, 85, 168–188. https://doi.org/https://doi.org/10.1016/j.jbi.2018.07.015.

(31) 	Walt, S. van der; Colbert, S. C.; Varoquaux, G. The NumPy Array: A Structure for Efficient Numerical Computation. Comput. Sci. Eng. 2011, 13 (2), 22–30. https://doi.org/10.1109/MCSE.2011.37.

(32) 	McKinney, W. Data Structures for Statistical Computing in Python. In Proceedings of the 9th Python in Science Conference; van der Walt, S., Millman, J., Eds.; 2010; pp 51–56.

(33) 	Brereton, R. G.; Jansen, J.; Lopes, J.; Marini, F.; Pomerantsev, A.; Rodionova, O.; Roger, J. M.; Walczak, B.; Tauler, R. Chemometrics in Analytical Chemistry—Part I: History, Experimental Design and Data Analysis Tools. Anal. Bioanal. Chem. 2017, 409 (25), 5891–5899. https://doi.org/10.1007/s00216-017-0517-1.

(34) 	Wei, Q.; Dunbrack Jr, R. L. The Role of Balanced Training and Testing Data Sets for Binary Classifiers in Bioinformatics. PLoS One 2013, 8 (7), 1–12. https://doi.org/10.1371/journal.pone.0067863.

(35) 	Gromski, P. S.; Muhamadali, H.; Ellis, D. I.; Xu, Y.; Correa, E.; Turner, M. L.; Goodacre, R. A Tutorial Review: Metabolomics and Partial Least Squares-Discriminant Analysis - a Marriage of Convenience or a Shotgun Wedding. Anal. Chim. Acta 2015, 879, 10–23. https://doi.org/10.1016/j.aca.2015.02.012.

(36) 	Devos, O.; Ruckebusch, C.; Durand, A.; Duponchel, L.; Huvenne, J. Chemometrics and Intelligent Laboratory Systems Support Vector Machines ( SVM ) in near Infrared ( NIR ) Spectroscopy : Focus on Parameters Optimization and Model Interpretation ☆. Chemom. Intell. Lab. Syst. 2009, 96 (1), 27–33. https://doi.org/10.1016/j.chemolab.2008.11.005.

(37) 	Grissa, D.; Pétéra, M.; Brandolini, M.; Napoli, A.; Comte, B.; Pujos-Guillot, E. Feature Selection Methods for Early Predictive Biomarker Discovery Using Untargeted  Metabolomic Data. Front. Mol. Biosci. 2016, 3, 30. https://doi.org/10.3389/fmolb.2016.00030.

(38) 	Krawczuk, J.; Łukaszuk, T. The Feature Selection Bias Problem in Relation to High-Dimensional Gene Data. Artif. Intell. Med. 2016, 66, 63–71. https://doi.org/10.1016/j.artmed.2015.11.001.

(39) 	Esbensen, K. H.; Geladi, P. Principles of Proper Validation: Use and Abuse of Re-Sampling for Validation. J. Chemom. 2010, 24 (3‐4), 168–187. https://doi.org/10.1002/cem.1310.

(40) 	Li, Z.; Li, L.; Pan, G.; Chen, J. Bioavailability of Cd in a Soil-Rice System in China: Soil Type versus Genotype Effects. Plant Soil 2005, 271 (1–2), 165–173. https://doi.org/10.1007/s11104-004-2296-7.

(41) 	Wang-da, C.; Guo-ping, Z.; Hai-gen, Y. A. O.; Wei, W. U.; Min, X. U. Genotypic and Environmental Variation in Cadmium , Chromium , Arsenic , Nickel , and Lead Concentrations in Rice Grains *. J. Zhejiang Univ. Sci. B 2006, 7 (7), 565–571. https://doi.org/10.1631/jzus.2006.B0565.

(42) 	Chung, I. M.; Kim, J. K.; Lee, K. J.; Park, S. K.; Lee, J. H.; Son, N. Y.; Jin, Y. I.; Kim, S. H. Geographic Authentication of Asian Rice (Oryza Sativa L.) Using Multi-Elemental and Stable Isotopic Data Combined with Multivariate Analysis. Food Chem. 2018, 240, 840–849. https://doi.org/10.1016/j.foodchem.2017.08.023.

(43) 	Zhang, Y.; Song, Q.; Yan, J.; Tang, J. Mineral Element Concentrations in Grains of Chinese Wheat Cultivars. 2010, 303–313. https://doi.org/10.1007/s10681-009-0082-6.