Target: Journal of Agricultural and Food Chemistry (IF: 3.571)

Determination of geographical origin by multi-element profiling combined with machine learning techniques: a study on Chinese Geographical Indication (GI) rice

Fei Xu1, Fanzhou Kong1, Hong Peng1, Shuofei Dong2, Weiyu Gao1, Guangtao Zhang1*

1Mars Global Food Safety Center, Beijing 101407, China

2Agilent Technologies (China) Co., Ltd, Beijing 100102, China

*Corresponding author: Tel.: +86 13331152657. E-Mail: Guangtao.zhang@effem.com

ABSTRACT

This study aimed to develop a novel strategy to determine geographical origins of Chinese GI rice, based on multi-element profiling using inductively coupled plasma mass spectrometry (ICP-MS). One hundred and thirty-one samples from six types of Chinese GI rice were analyzed, with 80% of the data assigned as training set and the rest 20% as testing set. Two machine learning algorithms, support vector machines (SVM) and random forest (RF), along with feature selection (reliefF algorithm) were implemented to build classification models. Independent validation by testing set showed that with the information from only four elements (27Al, 85Rb, 10B and 23Na), both SVM and RF could lead to a prediction accuracy of 100%. The results demonstrate that  multi-element profiling combined with machine learning techniques is an effective strategy to determine the geological authenticity of rice.

KEYWORDS

rice, ICP-MS, Geographical Indication, machine learning, feature selection, chemometrics

INTRODUCTION

In the current world of globalization, geographical authentication of food materials has emerged as an important issue, which is crucial for both quality assurance and food safety1. The notion of geographical indication (GI) first originated from the 19th century in Europe, with the purpose of protecting industrial property rights2. Nowadays, products with GI certification generally possess given quality, good reputations or other characteristics attributable to geographical origins3, thus making it possible for them to be differentiated from competitors’. According to the European Union (EU) quality schemes, three different systems have been enforced to protect the origin integrity of agroproducts and food: protected designation of origin (PDO), protected geographical indication (PGI) and traditional specialties guaranteed (TSG)4. Currently in China, three government sectors  supervise and protect GIs from different aspects at the administrative level5, including the State Administration for Industry and Commerce/ the Trademark Office (SAIC/TMO), the General Administration of Quality Supervision, Inspection and Quarantine (AQSIQ), and the Ministry of Agriculture (MoA).

Rice (Oryza sativa L.) is among the world’s top three largest food crops and is a staple food for nearly 50% of the world population6. China is leading in the rice paddy production in the world, with 220 million metric tons in 20187.With the improvement of people’s living standard, there is a growing domestic demand for rice with traceable origins8. However, due to the gap between the limited production and high market demand, GI rice has become more and more vulnerable to adulteration such as partial substitution and fraudulent labeling9,10. Therefore, determination of geographical origins of rice is of great importance for protecting the rights of farmers, retailers and consumers3.

Recent years, inductively coupled plasma mass spectrometry (ICP-MS) analysis in combination with multivariate analysis (MVA) and machine learning (ML) techniques, have been vigorously developed to address the issue of geographical authentication of rice11–13. ICP-MS is capable to simultaneously detect both metal and non-metal elements, and is characterized by high throughput measurement with wide dynamic range and relatively simple sample preparation14. MVA has been generally used to process and integrate data with large volume, in which principal component analysis (PCA) and linear discriminate analysis (LDA) are the two dominant methods due to their simplicity in spotting hidden trend embedded in the dataset. However, with the increase of data complexity and data volume, more advanced models based on pattern recognition are very much desired17. In the past decade, ML has demonstrated its capability to process complex problems, particularly in the domain of ecology18, biomedical19,20, astronomy21 and bioinformatics22. Yet so far, only a few studies have explored the application of ML in geographical authentication12,23.

The aim of this study was to develop a novel ML-based workflow for the determination of geographical origins of six types of Chinese GI rice. Support vector machines (SVM) and random foresee (RF) were utilized to uncover the hidden information in the element profiling obtained by ICP-MS, and thereby to construct reliable prediction models. Furthermore, feature selection was also applied to identify biomarkers that contributed the most to the differentiation between GI rices.

MATERIALS AND METHODS

Rice samples

In this study, a total of one hundred and thirty-one Chinese GI rice samples were directly collected from credible rice processing factories in five provinces in China (Heilongjiang, Liaoning, Jiangsu, Hubei and Guangxi). For simplicity’s sake, we named these samples as WC, PJ-1, PJ-2, SY, JS and GG. An overview of the geographical information of the samples is shown in Fig. 1.

Fig.1. Geographical information of GI rice samples collected from different regions in China with corresponding sample size.

Reagents and standards

Nitric acid (69%, part# 100441) was purchased from Merck Millipore (Darmstadt, Germany). Deionized water (18.3MΩ cm) was obtained from a Milli-Q system (Millipore, MA, USA). Multi-element calibration standard 2A (part# 8500-6940) and 4 (part# 8500-6942), environmental calibration standard (part# 5183-4688) and standard solutions of 45Sc (part# 5190-8578) and 103Rh (10 mg·L-1, part# 8500-6945) were purchased from Agilent Technologies (Santa Clara, CA, USA). The certified reference material (CRM) of rice flour (1568b) was purchased from the National Institute of Standards and Technology (Gaithersburg, MD, USA).

ICP-MS analysis

Firstly, 0.5 g of rice grains was weighed in a Teflon digestion vessel and mixed with 6mL of nitric acid. The vessel was placed in a fume hood overnight for pre-digestion and then transferred to the microwave oven (Anton Paar, Austria) for acid digestion. The digestion temperature of 180 °C was gradually reached in 15 min and held for 20 min. After digestion, the solution was cooled down to room temperature and diluted to 50mL with deionized water. To avoid cross contamination, all materials including the digestion vessels were soaked in a nitric acid solution (30% in water, v/v) for 24h and rinsed with deionized water for three times.

The concentrations of 30 elements (10B, 23Na, 24Mg, 27Al, 39K, 43Ca, 45Sc, 48Ti, 51V, 52Cr, 55Mn, 56Fe, 59Co, 60Ni, 65Cu, 66Zn, 70Ga, 73Ge, 75As, 78Se, 85Rb, 86Sr, 93Nb, 98Mo, 107Ag, 114Cd, 133Cs, 138Ba, 201Hg, 208Pb) were measured with Agilent 7900 ICP-MS (Agilent technologies, Santa Clara, CA, USA). The instrumental setting and operative conditions were adopted from a published method24 with some modifications: radio frequency power of 1550 W, radio frequency matching of 1.85 V and carrier gas flow rate of 1.05 L·min-1. The 103Rh standard solution was used as an online internal standard solution, to correct matrix effects and compensate for the possible instrumental deviations. The accuracy of analysis was verified by analyzing the rice flour CRM. Each sample was analyzed in duplicate.

Statistical analysis

To preliminarily evaluate the concentrations of the 30 elements in each GI rice sample, the original dataset was analyzed by one-way analysis of variance (ANOVA) coupled with Tukey’s test (p < 0.05). The dataset was then scaled through logarithmic transformation and subjected to unsupervised PCA for initial visualization. Two machine learning algorithms, RF and SVM were implemented to construct classifiers respectively. RF was first introduced by Breiman25, and it is made of an ensemble of decision trees which are generated from original dataset using bootstrap partition. SVM implements classifications by projecting input vectors into a high dimensional space thus to find a hyperplane that could separate different classes26. Feature selection is a data processing technique for data mining, which aims to identify pertinent features while discard irrelevant ones that are not informative but contribute to the overall dimensionality of the problem space27. In our study, ReliefF28,29 was utilized to select features through investigating the relative importance based on a calculated proxy statistic29.Fig. 2 showed the workflow for the training of classifiers and the validation of classification models:

      - The scaled dataset was randomly splitted into a training set (sample number: 104) and a testing set (sample number: 27) in a stratified fashion (80:20).

      - The ReliefF algorithm was applied to the training set and all the 30 features were ranked based on their differentation power. Subsequently, step-wise forward selection30 was conducted following with a 10-foldcross validation (grid search). The hyperpartmeters were xx. Eventually, the best combination of feature subsets and hyperparameters were used to construct optimal classifiers. Hyperparameters been tested were shown in Table S1.

      - The optimal classifiers were then independently validated on the testing set, and the prediction accuracy was determined.

Fig.2. Diagram of the machine learning based workflow for model training and validation.

All data analysis were carried out by R (R Core Team, 2019), RStudio (v3.5.1, Boston, MA, USA) and Python (v3.7, Python Core Team) with additional packages: dplyr (R)31, factoextra (R)32, FSelector (R)32, sklearn (Python)33, skrebate (Python)34, numpy (Python)35 and pandas (Python)36 .

RESULTS AND DISCUSSION

Elemental concentrations

As shown in Table S2, the measured concentrations of elements in CRM agreed well with the certified values, indicating the high accuracy of the ICP-MS analysis. Table 1 shows the measured concentrations of 30 elements in the six typies of Chinese GI rice. Overall, except for 208Pb, significant differences could be observed among all elements across all types of rice. However, it was not obvious which element contributed the most to differentiate all types of rice.

PCA analysis

As shown in Fig. 3a, the 1st and 2nd principal component (PC) were accountable for 60.7% of the total variance, and a clear separation was observed among PJ-1, GG and the rest types. While for JS, PJ-2, SY and WC, no satisfactory separation could be achieved. The loading plot (Fig. 3b) showed that 27Al, 70Ga, 93Nb,51V and 48Ti primarily contributed to the variations on PC1, while 23Na, 45Sc, 85Rb, 133Cs and 114Cd contributed to both PC1 and PC2. Notably, PJ-1 and PJ-2 could be clearly separated, despite that they were from the same geological origin (Fig. 3a). A possible explaination could be that cultivar types could also contribute to the elemental composition in rice kernals11.

Determination of geographical origins

High quality sampling is fundamental to achieve reliable results from multivariate model building37, while sample scaricity along with unrepresentative sampling are major reasons leading to unreliable classification38. In this study, rather than sampling from the market, we obtained all GI rice samples from reliable sources, which ensured the authencitity of samples and minimized the risk of modeling with “contaminated” dataset.

To date, ML algorithms such as support vector machines (SVM) and random forest (RF), have been reported to outperform traditional MVA39 and lead to predition models with increased reliability and robustness12,40. Fig. 4 showed the key results obtained from the training of classifiers. Beyond the function of dimention reduction before performing machine learning, feature selection is also capable to identify key features with high predictability, namely biomarkers41. The relative importance of each of the 30 elements is shown in Fig. 4a, indicating that 27Al, 85Rb, 10B, 23Na, and 86Sr were the leading elements that contributed the most to the differentiation of all types of GI rice. With only one selected feature (27Al), the mean cross-validation accuracy of 48% and 63% were achieved for RF and SVM, respectively (Fig. 4b). The performance of both RF and SVM boosted significantly with more features been added. Eventually, with only four features (27Al, 85Rb, 10B, and 23Na), the mean cross-validation accuracy of 100% was obtained by both RF and SVM with optimal hyperparameters applied (Fig. 4b). Notably, by applying feature selection solely on the training set instead of the entire dataset, we eliminated the risk of selection bias42.

While cross-validation was applied to assess the goodness-of-fit of modeling within the training set, it lacks the ability to represent the future working situation of the particular prediction model43. According to the Principle of Proper Validation (PPV)43, independent validation using a separate dataset  (testing set), is the one and only valid paradigm43 for model assessment. The result of independent validation using the testing set is shown in table 2. According to the result, with the information from only 4 selected elements (out of 30 elements been tested), both classifiers (SVM and RF) could predicit the geographical originis of all six types of GI rice with 100% accuracy. This indicated the success of the framework we have established, especially in constructing reliable prediction models while simultaneously identified potential biomarkers responsible for the differentiation. In a similar study where feature selection was also applied, it was also found that a small goup of elements (14Cd, 85Rb, 12Mg and19K) were the most relevant for the differientiation between rice samples obtained from two geological orgins in Brazil12. To further visualize the different element patterns of  six types of GI rice, we plotted the  relative median concentrations of four selected elements in radar plots. As shown in Fig. 5, the element profile of each GI rice was significantly different. Interestingly, the level of 27Al varied significantly between PJ-1 and PJ-2, which were sampled from the same geological location: the highest level of 27Al was found in PJ-1,  while the least concentration was found in PJ-2. Besides, the elements of 23Na, 85Rb, and 10B, all showed considerably different composition in PJ-1 and PJ-2. Such observation agreed with previous findings that, cultivar types also majorly impact on the element composition in rice44,45. Overall, it remains a challenging task to elucidate the rationale for why in our case, the four elements showed strong differentiation power. The complexity here, shall partially attribute to the sample diversity. In this study, we collected samples from all three dominate rice producing regions in China, including the northeast China plain (WC, PJ-1, and PJ-2), Yangtze River Basin (SY, JS), and southeast coastal region (GG). Such wide geological sampling scope, introduced multiple layers of variables (e.g. soil characteristics, agricultural practices, and genotype variation), which are all closely related to the element profile of crops46,47.

Beyond the four selected elements  (27Al, 85Rb, 10B, and 23Na), the element of 114Cd also drew our attention. In a study on Brazilian rice12 mentioned previoulsy, it was found that the level of 114Cd alone can be used to differentiate rice from two geological origins. The author further pointed out that it was the difference in irrigation methods applied to paddy filed, that resulted in the variance of 114Cd composition in rice. According to our results, 114Cd was found in all six types of GI rice, with the GG rice, sampled from Guangxi province, having the highest level (Table 1). To test the efficiency of the ML-based framework we established, we first regrouped the original dataset as GG rice and non-GG rice. According to the result of feature selection, 114Cd was identified as the element with the most differentiation power over the two subgroups. Ultimately, the independent validation by testing set confirmed that 114Cd along can be used to differentiate GG rice and non-GG rice, with 100% predication accuracy.Fig. 6 visualized the significant difference of 114Cd concentration between GG and non-GG rice samples (P < 0.05). A previous national scale study showed that the concentration of 114Cd in paddy soils from different Chinese regions varied significantly, majorly due to the combination of geogenic factors and anthropogenic pollutions (e.g. mining)48. Particularly, the highest level of 114Cd can be found in southeast coastal regions including Hunan and Guangxi province48. With this, we concluded that by applying the ML-based workflow , GG rice can be differentiated from all other GI rice, with the information provided by 114Cd alone. To further protect GI rice from potential fraudulent activities, “positive” samples (e.g. GI rice mixed with serial doses of “look-alikes”49) shall be introduced into the classification in future studies.

Our study demonstrated that multi-element profiling using ICP-MS coupled with ML techniques, could differentiate six types of Chinese GI rice with extremely high accuracy. Particularly, we identified four elements with the most differentiation power. This opens the door for future study on the application of fast, and low-cost multi-element profiling tools such as X-ray fluorescence (XRF) spectrometry50, in the determination of rice geological origins.

ABBREVIATIONS USED

ANOVA, one-way analysis of variance; AQSIQ, General Administration of Quality Supervision, Inspection and Quarantine; CRM, certified reference material; EU, European Union; GI, geographical indication; ICP-MS, inductively coupled plasma mass spectrometry; LDA, linear discriminate analysis; ML, machine learning; MoA, Ministry of Agriculture; MVA, multivariate analysis;  PC, principal component; PCA, principal component analysis; PDO, protected designation of origin; PGI, protected geographical indication; RF, random forest; SAIC, State Administration for Industry and Commerce; SVM, support vector machines; TMO, the Trademark Office; TSG, traditional specialties guaranteed

ACKNOWLEDGMENT

We want to thank Dr. Di Wu from Yangtze Delta Region Institute of Tsinghua University for his tremendous support on sampling. We thank Ms. Si Lin and Hongwei Qiao for their industrious work on  experimentation and documentation.

FUNDING SOURCES

TBC

REFERENCE

1.	Drivelos, S. A. & Georgiou, C. A. Multi-element and multi-isotope-ratio analysis to determine the geographical origin of foods in the European Union. TrAC - Trends Anal. Chem. 40, 38–51 (2012).

2.	Summary of the Paris Convention for the Protection of Industrial Property (1883). Available at: https://www.wipo.int/treaties/en/ip/paris/summary_paris.html.

3.	Luykx, D. M. A. M. & Ruth, S. M. Van. Food Chemistry An overview of analytical methods for determining the geographical origin of food products. 107, 897–911 (2008).

4.	PDO and PGI Agricultral products:a 14.2 billion euro turnover for over 800 products. Available at: https://ec.europa.eu/agriculture/quality/schemes/newsletter-2010_en.pdf.

5.	Li, Y. Protection of Geographical Indications in China. (2017). Available at: https://www.niuyie.com/protection-of-geographical-indications-in-china/.

6.	Chauhan, B. S., Jabran, K. & Mahajan, G. Rice Production Worldwide.

7.	Paddy rice production worldwide 2017-2018, by country,. Statista (2019). Available at: https://www.statista.com/statistics/255937/leading-rice-producers-worldwide/.

8.	Jin, S., Zhang, Y. & Xu, Y. Amount of information and the willingness of consumers to pay for food traceability in China. Food Control (2017). doi:10.1016/j.foodcont.2017.02.012

9.	Rodriguez, L., Hall, B., Avenue, S. G., Hall, G. & Street, S. W. Social trust and risk knowledge , perception and behaviours resulting from a rice tampering scandal Sela Sar. 5, 80–96 (2014).

10.	Veeck, G. Food Safety Concerns and Rice Imports in China : 1998 - 2016. (2017).

11.	Cheajesadagul, P., Arnaudguilhem, C., Shiowatana, J., Siripinyanond, A. & Szpunar, J. Discrimination of geographical origin of rice based on multi-element fingerprinting by high resolution inductively coupled plasma mass spectrometry. Food Chem. 141, 3504–3509 (2013).

12.	Maione, C., Batista, B. L., Campiglia, A. D., Barbosa, F. & Barbosa, R. M. Classification of geographic origin of rice by data mining and inductively coupled plasma mass spectrometry. Comput. Electron. Agric. 121, 101–107 (2016).

13.	Wang, X. & Harrington, P. de B. Differentiating Rice Varieties by Inductively Coupled Plasma Mass Spectrometry Chemical Profiling with Singular Value Decomposition Background Correction. J. Anal. Test. 2, 138–148 (2018).

14.	Wadood, S. A., Boli, G., Xiaowen, Z., Hussain, I. & Yimin, W. Recent development in the application of analytical techniques for the traceability and authenticity of food of plant origin. Microchem. J. 152, 104295 (2020).

15.	Bevilacqua, M. et al. Recent chemometrics advances for foodomics. TrAC Trends Anal. Chem. 96, 42–51 (2017).

16.	Maione, C. & Barbosa, R. M. Recent applications of multivariate data analysis methods in the authentication of rice and the most analyzed parameters: A review. Crit. Rev. Food Sci. Nutr. 8398, 1–12 (2018).

17.	Jiménez-Carvelo, A. M., González-Casado, A., Bagur-González, M. G. & Cuadros-Rodríguez, L. Alternative data mining/machine learning methods for the analytical evaluation of food quality and authenticity – A review. Food Research International 122, (Elsevier Ltd, 2019).

18.	Cutler, D. R. et al. RANDOM FORESTS FOR CLASSIFICATION IN ECOLOGY. Ecology 88, 2783–2792 (2007).

19.	Hu, W. Identifying predictive markers of chemosensitivity of breast cancer with random forests. J. Biomed. Sci. Eng. 3, (2010).

20.	Ram, M., Najafi, A. & Shakeri, M. T. Classification and Biomarker Genes Selection for Cancer Gene Expression Data Using  Random Forest. Iran. J. Pathol. 12, 339–347 (2017).

21.	Gao, D., Zhang, Y. & Zhao, Y. Random forest algorithm for classification of multiwavelength data. Res. Astron. Astrophys. 9, 220 (2009).

22.	Boulesteix, A.-L., Janitza, S., Kruppa, J. & König, I. R. Overview of random forest methodology and practical guidance with emphasis on computational biology and bioinformatics. WIREs Data Min. Knowl. Discov. 2, 493–507 (2012).

23.	Richter, B., Rurik, M., Gurk, S., Kohlbacher, O. & Fischer, M. Food monitoring: Screening of the geographical origin of white asparagus using FT-NIR and machine learning. Food Control 104, 318–325 (2019).

24.	Hopfer, H., Nelson, J., Collins, T. S., Heymann, H. & Ebeler, S. E. The combined impact of vineyard origin and processing winery on the elemental profile of red wines. Food Chem. 172, 486–496 (2015).

25.	Breiman, L. Random Forests. Mach. Learn. 45, 5–32 (2001).

26.	Cortes, C. & Vapnik, V. Support-Vector Networks. Mach. Learn. 20, 273–297 (1995).

27.	Urbanowicz, R., Meeker, M., LaCava, W., Olson, R. & Moore, J. Relief-Based Feature Selection: Introduction and Review. J. Biomed. Inform. 85, (2017).

28.	Dietterich, T. Machine Learning Research: Four Current Directions. AI Mag. 18, (2000).

29.	Relief-based feature selection: Introduction and review. J. Biomed. Inform. 85, 189–203 (2018).

30.	Data Handling in Science and Technology. in Wavelets in Chemistry (ed. Walczak, B.) 22, ii (Elsevier, 2000).

31.	Wickham, H., François, R., Henry, L. & Müller, K. dplyr: A Grammar of Data Manipulation. (2019).

32.	Mundt, A. K. and F. factoextra: Extract and Visualize the Results of Multivariate Data Analyses. (2017).

33.	Pedregosa, F. et al. Scikit-learn: Machine Learning in {P}ython. J. Mach. Learn. Res. 12, 2825–2830 (2011).

34.	Urbanowicz, R. J., Olson, R. S., Schmitt, P., Meeker, M. & Moore, J. H. Benchmarking relief-based feature selection methods for bioinformatics data mining. J. Biomed. Inform. 85, 168–188 (2018).

35.	Walt, S. van der, Colbert, S. C. & Varoquaux, G. The NumPy Array: A Structure for Efficient Numerical Computation. Comput. Sci. Eng. 13, 22–30 (2011).

36.	McKinney, W. Data Structures for Statistical Computing in Python. in Proceedings of the 9th Python in Science Conference (eds. van der Walt, S. & Millman, J.) 51–56 (2010).

37.	Brereton, R. G. et al. Chemometrics in analytical chemistry—part I: history, experimental design and data analysis tools. Anal. Bioanal. Chem. 409, 5891–5899 (2017).

38.	Filzmoser, P. & Group, F. Intro. to multivariate statistical Analysis in Chemometrics. (2008).

39.	Gromski, P. S. et al. A tutorial review: Metabolomics and partial least squares-discriminant analysis - a marriage of convenience or a shotgun wedding. Anal. Chim. Acta 879, 10–23 (2015).

40.	Devos, O., Ruckebusch, C., Durand, A., Duponchel, L. & Huvenne, J. Chemometrics and Intelligent Laboratory Systems Support vector machines ( SVM ) in near infrared ( NIR ) spectroscopy : Focus on parameters optimization and model interpretation ☆. Chemom. Intell. Lab. Syst. 96, 27–33 (2009).

41.	Grissa, D. et al. Feature Selection Methods for Early Predictive Biomarker Discovery Using Untargeted  Metabolomic Data. Front. Mol. Biosci. 3, 30 (2016).

42.	Krawczuk, J. & Łukaszuk, T. The feature selection bias problem in relation to high-dimensional gene data. Artif. Intell. Med. 66, 63–71 (2016).

43.	Esbensen, K. H. & Geladi, P. Principles of Proper Validation : use and abuse of re-sampling for validation. 168–187 (2010). doi:10.1002/cem.1310

44.	Li, Z., Li, L., Pan, G. & Chen, J. Bioavailability of Cd in a soil-rice system in China: Soil type versus genotype effects. Plant Soil 271, 165–173 (2005).

45.	Wang-da, C., Guo-ping, Z., Hai-gen, Y. A. O., Wei, W. U. & Min, X. U. Genotypic and environmental variation in cadmium , chromium , arsenic , nickel , and lead concentrations in rice grains *. 7, 565–571 (2006).

46.	Chung, I. M. et al. Geographic authentication of Asian rice (Oryza sativa L.) using multi-elemental and stable isotopic data combined with multivariate analysis. Food Chem. 240, 840–849 (2018).

47.	Zhang, Y., Song, Q., Yan, J. & Tang, J. Mineral element concentrations in grains of Chinese wheat cultivars. 303–313 (2010). doi:10.1007/s10681-009-0082-6

48.	Liu, X., Tian, G., Jiang, D., Zhang, C. & Kong, L. Cadmium (Cd) distribution and contamination in Chinese paddy soils on national scale. Environ. Sci. Pollut. Res. 23, 17941–17952 (2016).

49.	Yu, Y. et al. Accuracy and stability improvement in detecting Wuchang rice adulteration by piece-wise multiplicative scatter correction in the hyperspectral imaging system. Anal. Methods 10, 3224–3231 (2018).

50.	Worku, M. et al. Differentiating the geographical origin of Ethiopian coffee using XRF- and ICP-based multi-element and stable isotope profiling. Food Chem. 290, 295–307 (2019).